Technical Report: Scalable NxN Systolic Array Matrix Multiplier

1. Project Overview

This project implements a hardware-based matrix multiplier using a systolic array architecture. The design is optimized for parallel, pipelined computation, passing data rhythmically through Processing Elements (PEs) to perform C = C + A x B.

2. Architecture Design

2.1 Processing Element (PE)

The PE is the fundamental unit of the array, designed to perform a Multiply-Accumulate (MAC) operation.

- MAC Operation: C = C + A x B.

- Pipelining: Data is registered on every clock cycle to allow simultaneous operations across the grid. a_out, b_out are for pipeline.

- Data Flow: a_in and b_in are passed to a_out and b_out respectively, ensuring local data movement between neighbors.

2.2 Systolic Array Top Module

The top module organizes N x N PEs into a grid.

- Hierarchical Structure: To satisfy the automated marking scheme, rows are named pe_row[i], columns are pe_col[j], and PEs are u_pe.

- Interconnectivity: Vertical and horizontal wires connect the output of one PE to the input of the next.

- Scalability: The design uses Verilog generate blocks to scale from 2x2 up to 8x8.

3. Pipeline Stages and Timing

Data must be "skewed" before entering the array to account for the pipeline registers.

- Stage 1 (Row 0, Col 0): Starts at T=0.

- Stage 2 (Neighbors): PEs at (0,1) and (1,0) receive data at T=1.

- Stage 3 (Diagonal): The bottom-right PE (1,1) receives its first valid data at T=2

- Latency: The total computation time depends on the size N. The Done signal is triggered once the pipeline is flushed and all results are accumulated.

Total clock cycles required = 2N-1

First output appears after Ncycles

Last output appears at 2N-1cycles

4. Simulation Results

- Small Integers: Verified 2 x 2 multiplication with known values.

- Random Stress: Verified accuracy using a self-checking testbench against a golden software model.